BASIC11 Language Interpreter for the 68HC11EVB

Documentation & User's Manual

==============================================

COPYRIGHT NOTICE

The entire contents of this manual and the software described

herein are copyrighted with all rights reserved.  No part of this manual

or the software may be copied in whole or in part without the express

permission of the author.

WARRANTY

Even though many hours of work went into the writing and testing

of BASIC11 and it is believed to be "bug free", BASIC11 is supplied

AS-IS and without warranty.  The author makes no express or implied

warranties as to the fitness of use and merchantability of the product.

The user assumes the entire risk as to its quality, performance and

fitness of use.

In no event will the author be liable for direct, indirect,

incidental, or consequential damages resulting from the use of this

product.  Including but not limited to loss of sales, income, service,

profits, or potential profits.

In the event a situation is found where the program does not

function as the manual describes, the author will attempt to correct any

errors brought to his attention, however he makes no guarantee to do so.

===============================================================================

1. Introduction

BASIC11 is a very fast and complete control oriented BASIC

interpreter for the Motorola MC68HC11 single chip microcomputer. It

provides all the functions of standard BASIC along with a number of

enhancements that allow direct control of some of the MC68HC11's

hardware features using BASIC statements.

The only limitations of BASIC11 (which usually aren't

limitations in a control environment) are that it only supports integer

variables.  Also  strings are only supported in PRINT and INPUT

statements.

Lines entered into a BASIC11 program must begin with a line

number and must be terminated by a carriage return.  Lines may be no

longer than 80 characters.  All lines are automatically put in numerical

order by BASIC11 as they are typed in.  Lines may be deleted from a

program by simply typing the line number followed immediately by a

carriage return.

The syntax of each line in a BASIC11 program is checked as soon

as a CARRIAGE RETURN is entered and any errors are reported immediately.

This prevents the interpreter from having to check syntax at runtime and

is one of the things that contributes to BASIC11's speed.

2. The Basics of BASIC11

2.1 Lines

Each line of a BASIC11 program must begin with a line number.

Lines may be numbered from 1 through 32767 and each line must be

terminated by a CARRIAGE RETURN. Lines may contain multiple statements

that are separated by colons. Spaces may be used freely in BASIC11

statements to improve their readability with one exception.  Assignment

statements and arithmetic/logic statements may contain no imbeded

blanks.  Some examples follow:

10 PRINT X,   X*X  ,   RND(0)-5

20 X=5  :  Y=10  : Z=15

2.2 Integer Constants:

All integer constants are represented internally as 16 bit two's

complement numbers with a decimal range of -32768 to 32767.  In the

source program and input statements numbers may represented in either

decimal or hexidecimal form.  All hexidecimal constants must be prefixed

by a dollar sign ($). Some examples of integer constants are:

50 X=1000

60 Y=-55

70 Z=PEEK($E010)

2.3 String Constants:

As mentioned earlier, BASIC11 does not support string

variables.  However, it does support string constants in both PRINT

statements and INPUT statements to allow for prompting of input data.

Some examples of string constants follow:

100 PRINT "Please Enter Your Name"

200 INPUT "Enter a Number",N

2.4 Variables:

BASIC11 currently supports only integer variables (it may

support string and floating point variables in the future).  Integer

variable names can consist of a single alphabetic letter or a letter

followed by another letter or number.  Examples of integer variable

names are:

AB, XZ,R1,TO,IF

Notice in the above example that two of the variables are the

same as the BASIC11 keywords TO and IF.  In many BASIC's this is illegal

but in BASIC11 it is perfectly legal.

Any legal integer variable name may also be subscripted or

dimensioned using the DIM statement. A variable is dimensioned by

following any legal integer variable name by an expression that is

enclosed in parentheses.  Note that when a variable is declared in a DIM

statement storage is not allocated until runtime.  This is because all

array storage is allocated dynamically.  All dimensioned variables start

with 0. For example:

300 DIM AX(4)

Will create the following five variables:

AX(0), AX(1), AX(2), AX(3), AX(4)

Again, the same variable name may be used for both a

non-dimensioned and dimensioned variable. All dimensioned variables must

be declared in a DIM statement before they can be referenced in an

expression or Error # 24 will result when the variable is referenced

during a program run.

2.5 Variable Assignment

By using the LET, INPUT, INBYTE or the READ statements variables

may be assigned values. The most common way to assign a value to a

variable is through the use of the LET statement.  For example, the

statement:

90 LET GD=7

Would assign the integer value of 7 to the variable "GD" so that each

time the variable "GD" was used in an expression, the numeric value of 7

would actually be substituted.

An INPUT statement, when executed, will cause BASIC11 to stop,

print a question mark on the terminal, and wait for the user to enter a

numeric constant.  For example, the statement:

40 INPUT A1

will assign whatever number is typed at the terminal to the variable

"A1".

The INBYTE statement is similar to the input statement except

that instead of expecting an ASCII formatted number from the specifyed

input device, it assigns the value of the ASCII byte to the variable

that follows it.  For example if the statement INBYTE AX were executed

and the character "Y" were typed at the terminal, the variable AX would

contain the value 89 which is the numeric value of the ASCII character

"Y".  The INBYTE statement is very useful for obtaining data from input

devices other than the control terminal.

The READ statement works almost like the INPUT statement except

that the numeric constant is taken from a DATA statement instead of

being typed in by a user from the terminal (more about the READ and DATA

statements later).

2.6 Operators:

There are three classes of operators available in BASIC11. The

one most are familiar with is the mathematical operators.  Addition,

subtraction, multiplication, and division. The mathematical operators

are:

SYMBOL	EXAMPLE	MEANING

+	A+B	Add A to B

-	A-B	Subtract B from A

*	A*B	Multiply A and B

/	A/B	Divide A by B

\	A\B	Remainder of (A/B)

The next class of operators is the logical operators.  They are

used to perform "bitwise" operations. They can be used to "ignore"

certain bits within a word or in conditional tests when more than one

condition needs to be tested.  The logical operators are:

SYMBOL	EXAMPLE	MEANING

.AND.	A.AND.B	Bitwise logical AND of A and B.

.OR.	A.OR.B	Bitwise logical OR of  A and B.

.EOR.	A.EOR.B	Bitwise logical EXCLUSIVE OR of A and B.

The last class of operators is the relational operators.  These

are used in the IF and WHILE statements to test whether one expression

is less than, greater than, or equal to another expression.  The

relational operators are:

SYMBOL	EXAMPLE	MEANING

=	A=B	True if A is equal to B

<>	A<>B	True if A is not equal to B

<	A<B	True if A is less than B

>	A>B	True if A is greater than B

<=	A<=B	True if A is less than or equal to B

>=	A>=B	True if A is greater than or equal to B

2.7 Operator Precedence:

Overall operator precedence is shown below.  The operator at the

top of the list has the highest priority in any expression, while the

operator at the bottom has the lowest priority.

()	Expressions enclosed in parenthesis

-    NOT	Unary minus and NOT (one's complement)

*   /   \	Multiplication, division, and Mod (remainder)

+   -		Addition and subtraction

=	Relational operators

<>

<

>

<=

>=

.AND.	All logical operators have the same precdence

.OR.

.EOR.

2.8 Operating Modes:

BASIC11 has two operating modes, the RUN mode and the immediate

Mode.  In the RUN mode program lines that have previously been entered

are executed starting with the smallest line number and continues until

a STOP or END statement is executed, an error occurs, or a control-C is

typed on the terminal.

In the immediate Mode, any legal BASIC11 statement or command

may be typed in without a line number and the statement will immediately

be executed.  BASIC11 may be used in this mode to debug programs by

examining variables, memory locations, or I/O ports.

2.9 Remarks:

It is a good idea to place remarks throughout your programs so

that someone else can understand the operation of your program if it

ever becomes necessary to change it.  It can even help you if you

haven't worked with the program in a while.  Even though the REM

statement is not executable it may be referenced by other program

statements (for example, by a GOTO or GOSUB statement).

3.0 Commands:

Commands are instructions to BASIC11 that allow it to perform

"housekeeping" tasks at the users request.  None of the following

commands may appear in a BASIC11 program.

NAME	EXAMPLE/EXPLANATION

CLEAR	CLEAR

The clear command is used to set all variables to zero and to

reset the GOSUB, WHILE, and FOR - NEXT stacks.  A clear is automatically

performed when a RUN command is entered.

CONT	CONT

The CONT command is used to restart a BASIC11 program either

after it has been stopped by either a STOP statement or a control-C was

typed at the terminal. The program can't be restarted if an error

occurred in the program or if the program is modified.

LIST	LIST	Lists the entire program

LIST [line #]	Lists one line

LIST [line #]-[line #]	Lists from the first line number

through the second line number

The LIST command can be used to display selected lines of the

program on the terminal. As can be seen from the above examples, all,

part, or a single line of the program may be listed.

LLIST	LLIST

LLIST [line #]

LLIST [line #]-[line #]

The LLIST works in the same manner as the LIST command, except

that the program lines are sent to the system printer instead of the

terminal.

NEW	NEW

The NEW command is used to clear out both the BASIC program

buffer and the variable storage space.  It prepares BASIC11 to accept a

"New" program.

RUN	RUN

The RUN command is used to begin execution of the program that

is currently in memory.

ESAVE	ESAVE

The ESAVE command is used to save the program that is currently

in RAM to the program storage EEPROM that resides in the system.

ELOAD	ELOAD

The ELOAD command is used to transfer a program to RAM that had

previously been saved using the ESAVE command.

AUTOST	AUTOST

The AUTOST command is used to set a flag that resides in the

program storage EEPROM that will allow the BASIC11 program to execute

from a powerup or reset condition.  Note that the BASIC11 program is

executed out of the program storage EEPROM and is not copied into RAM.

This allows the entire system RAM to be used for variable storage.

NOAUTO	NOAUTO

This command resets the auto start flag set by the AUTOST

command and disables the automatic execution of a BASIC program stored

in the program storage EEPROM.

4.0 Statements:

All of the following statements are used in the creation of

BASIC11 programs. The statements are arranged in logical groups to make

similar functions easy to find.  Each statement is accompanied by one or

more program lines showing it's proper usage and an explanation of how

the statement works if necessary.

4.1 Assignment:

DATA	<line number> DATA <number> [,<number>,<number>....]

10 DATA 500,-10,200,99,$CD03

20 DATA $FE   ,   1000,  -300

The data statement is used to specify data that will be assigned

to variables with a READ statement.  The data is read from left to right

and always begins with the first data statement in the program.  When

the program has read all the data in a single DATA statement, BASIC11

will search the program for the next DATA  statement starting at the

line following  the just exhausted DATA line.  This is done because all

data statements in a program are considered logically to be one long

DATA statement.

LET	<line number> LET <variable>=<expression>

10 LET X=5

20 LET Y=25*(Y/3)

30 LET AX(3)=AX(5)*10

40 CD=DE+23

50 XZ=-55

The LET statement is the most often used way to assign a value

to a variable. Notice in line numbers 40 and 50 above do not contain the

keyword LET.  This is what is known as an implied LET and is a feature

of BASIC11 to help cut down typing time when entering a program since

this is one of the most often used statements.  One final note. As

stated earlier, Assignment statements and arithmetic/logic statements

may contain no imbeded spaces.  This means that there may be no spaces

between the variable and equals, the equals and the start of the

expression, and no spaces within the expression.

READ	<line number> READ <variable> [,<variable>,<variable>,....]

READ A,B,C

The READ statement is used in conjunction with the DATA

statement to assign values to variables.  The first time the READ

statement is executed, it will assign the first item in the first DATA

statement to the first variable in its variable list.  If additional

variables are present in its variable list, each one will sequentially

be assigned the next item in the DATA statement.  Care must be taken

when a program is written so that BASIC11 does not try to read past the

last item in the last DATA statement.  If this happens, Error # 38 will

be issued.

RESTORE	<line number> RESTORE

330 RESTORE

The RESTORE statement is used to reset BASIC11's internal

"pointer to the next item" in a DATA statement to the first item in the

first DATA statement that appears in the program.

EEP()	<line number> EEP(<expression>)=<expression>

25 EEP(30)=$55

30 EEP(X+1)=A/B

The EEP() statement is actually a special form of the implied

LET.  EEP() is actually a subscripted variable that allows the BASIC

program to directly write to the MC68HC11's on board EEPROM.  All the

timing and control information necessary to write to the EEPROM is taken

care of by BASIC11.  This feature makes it very convenient to save any

kind of calibration data that must be retained in the event of a power

failure. Currently the subscript of the EEP() statement is limited to

255.  CAUTION: Since the the number of write/erase cycles of the EEPROM

is limited, be very careful that the EEP() statement doesn't get

executed repeatedly for the same location by having it reside within a

loop.

PORTA

PORTB

PORTC

PORTD	<line number> PORTx=<expression>

75 PORTA=$A5

85 PORTC=X+(E-K)

The PORTx statement is also a special form of the implied LET

statement.  It allows BASIC11 to directly assign an 8-bit value to one

of the MC68HC11's I/O ports. Note that for a logic value to actually

appear on one of the port pins, that particular pin must have been

programed as an output by using the POKE() statement to write a one to

that particular port's Data Direction Register (DDR).  If a value of

greater than 255 ($FF) is written to a port, Error #46 will be issued.

TIME	<line number>  TIME=<expression>

65  TIME=0

15  TIME=SC/60

The TIME statement, like the EEP() and PORT statement, is a

special form of the implied LET statement that allows the BASIC program

to assign a value to the system variable TIME which is used as BASIC11's

"Real Time Clock".  BASIC11 uses the output compare one (OC1) register

to generate a periodic interrupt which is then divided down by software

so that the variable TIME is incremented once per second.  Since the

variable is a 16 bit number, elapsed time can be kept track of for 65536

seconds (approximately 18 hours) without any software overhead.

PACC	<line number> PACC=<expression>

85  PACC=25

95  PACC=-5.AND.$00FF

Like the TIME, EEP(), and PORT statements, PACC statement is a

special form of the implied LET statement that allows the programmer to

directly alter the value of the MC68HC11s Pulse Accumulator.  Since the

Pulse Accumulator is only an eight bit register, the must be in the

range 0 � expression �255.

4.2 Control Transfer:

GOSUB	<line number> GOSUB <line number>

100 GOSUB 1000

The GOSUB statement is used to transfer control of the program

to the subroutine whos line number follows the GOSUB statement.  The

last statement of any subroutine should be a RETURN statement which will

return control back to the statement following the GOSUB.

RETURN	<line number> RETURN

1100 RETURN

As mentioned above the RETURN statement should be the last

executed statement in a subroutine and will return program execution to

the statement following the GOSUB.

GOTO	<line number> GOTO <line number>

50 GOTO 10

The GOTO statement is used  just to transfer control of program

execution to the line number following the GOTO statement.

ON  GOSUB	<line number> ON <expression> GOSUB <line number> [,<line

number>,......]

200  ON  X+1  GOSUB  10,90,300,550

The ON  GOSUB statement provides a facility to allow BASIC11 to

decide which of a number of subroutines to execute based on the value of

an expression.  When the expression is evaluated, the resulting number

is used to pick one of the line numbers following the GOSUB it should

execute.  In the above example if  X were equal to 0, the expression

would evaluate to 1 and the subroutine starting at line 10 would be

executed. If X were equal to 1, then the subroutine at line 90 would be

executed and so on. If the expression evaluates to 0, a negative number

or a number that is greater than the number of lines listed after the

GOSUB, Error #32 will be issued.

ON  GOTO	<line number> ON  GOTO <line number> [,<line number>,....<line

number>]

500  ON  X  GOTO 100,200,300,400,500

The ON  GOTO statement works in basically the same manner as the

ON  GOSUB except that control is transferred directly to the line number

that is selected from the list following the GOTO.  No return address is

saved and hence control cannot be returned to the statement following

the ON  GOTO statement.

4.3 Conditional Tests:

IF  THEN	<line number> IF <expression> THEN <line number>

55  IF  A=1   THEN  200

70  IF  A=1.AND.B=1  THEN 500

The IF  THEN statement is used to transfer control of the

program to another statement based on the results of the evaluation of

the expression.  If the expression is true (evaluates to any non-zero

value) then control is transferred to the statement at the line number

following THEN.  If the expression evaluates as false (equal to zero)

then the next sequential statement in the program will be executed.

Notice in the second example that multiple conditions may easily be

tested in a single IF statement by use of the logical operators.

IF THEN

ELSE	<line number> IF <expression> THEN <line number> ELSE <line

number>

75  IF  PORTA=$FE  THEN  200  ELSE  300

This form of the IF  THEN statement is a slight variation in

that if the expression is evaluated as false control of the program is

transferred to the line number following the ELSE clause.

NOTE: In the above examples a space follows  the expression in

the IF statement.  This IS REQUIRED so that BASIC11 will know  where the

expression ends.  Failure to follow the expression with a space will

result in an Error being reported (most likely Error #6).

4.4 Input/Output:

INPUT	<line number> INPUT ["string constant",] <variable>

[,<variable>,....<variable>]

45 INPUT "ENTER THREE NUMBERS", A, B ,C

55 INPUT XE, ZE, PI

The input statement is one of the ways that a value may be

assigned to a variable.  When the INPUT statement is executed, the

prompt string, if present, will be printed on the terminal followed by a

question mark and will wait for the user to enter the requested data. If

the user enters less data than is requested, BASIC11 will respond by

printing a question mark on the next line and will wait for the next

piece of data to be entered. This will continue until all requested data

has been entered by the user.  If more data is entered by the user than

was requested by the INPUT statement, the excess will be ignored.

Note that if the user responds to an INPUT statement with a

control-C, program execution will be halted and BASIC11 will return to

the command mode.  Note that the program cannot be restarted by the use

of the CONT command.

PRINT	<line number> PRINT [variable, expression, "string constant"]

10 PRINT  "THE VALUE OF X IS "; X

20 PRINT  X, X*X, X/Z+3

30 PRINT  X, Y, Z

35 PRINT  A, B, C;

65 PRINT

The PRINT statement may be optionally followed by any

combination of variables, expressions, or string constants each

separated by either a comma or semicolon. The significance of separating

the items in a PRINT statement by either a comma or a semicolon is

explained below.

BASIC11 divides each output line into "fields" of eight (8)

characters.  When the arguments following a PRINT statement are

separated by commas, BASIC11 will print each item beginning at the next

field in the line.  In line 30 in the above example, BASIC11 would print

the value of variable X beginning in column 0, the value of variable Y

would be printed starting in column 8 and the value of variable Z would

be printed starting in column 16.

separating variables with semicolons effectively disables this

"fielding" feature by printing variables and constants next to one

another.  There will still be a space or two between successive numeric

expressions that are printed because each number that is printed with

one trailing space.  Also if a number is not negative a space will be

printed in front of the number in place of the minus sign.

Notice in line number 35 above that a semicolon (it could have

been a comma) follows the last variable.  This has the effect of

suppressing the normal carriage return/line feed sequence that would

normally be issued after printing the last expression.

As mentioned in the first paragraph, the argument list that

follows the PRINT statement is optional as is illustrated in the example

of line 60 above.  This form of the print statement has the effect of

printing only a blank line.

?	<line number> ?  [variable, expression, "string constant"]

The question mark can be entered instead of the keyword "PRINT"

to save typing time when entering a program or executing a line in the

immediate mode.  When entered in a program line the question mark is

replaced by the same token as the keyword PRINT. Because of this, when

the program line is listed the keyword PRINT will appear instead of the

question mark.

INPUT#	<line number> INPUT #<expression,> ["string constant",]

<variable> [,<variable>,...<variable>]

15  INPUT #5, AX

20  INPUT #0, AX,DF,EG

25  INPUT #DE-1, LO

This form of the INPUT statement works in the same manner as the

standard INPUT statement except that the data is obtained from a

hardware device other than the control console or terminal.  The

expression that follows the "#" is evaluated and is then used to choose

one of eight (8) different hardware devices, numbered 0 through 7, from

which to get the input data.  Device numbers 0 and 1 are currently used

by BASIC11 for the control console and the system printer respectively.

All other device numbers are available to the user for inputing data

from special devices.  Note however that the user must provide the

proper "drivers" for each device.  For a detailed explanation of how to

use alternate devices for input, see Appendix A.  If <expression> is

negative or greater than seven(7) error number 48 will be issued.

PRINT #	<line number> PRINT #<expression,> [<expression>....<expression>]

25  PRINT #1,"THIS IS A TEST"

35  PRINT #0,AX,FG,R

This form of the PRINT statement works in the same manner as the

standard PRINT statement except that data is sent to an alternate

hardware device.  The same rules that apply to the INPUT # statement

also apply to the PRINT # statement.  Again, for a detailed explanation

of how to use alternate devices for output, see Appendix A.

INBYTE	<line number>  INBYTE  <variable>

10  INBYTE  DC

20  INBYTE #3,AX(Z)

30  INBYTE #1,CV

The INBYTE statement is another way that a value may be assigned

to a variable.  The INBYTE statement is similar to the input statement

except that instead of expecting an ASCII formatted number from the

specifyed input device, it assigns the value of an ASCII byte to the

variable that follows it.  If the statement in line 10 were executed and

the character "Y" were typed at the terminal, the variable DC would

contain the decimal value 89 which is the numeric value of the ASCII

character "Y".  The INBYTE statement is very useful for obtaining data

from input devices other than the control terminal. The example in line

number 30 above would retrieve a byte of data from the HC11's Serial

Communications Interface (SCI) and would assign that value to the

variable CV.

4.5 Looping Constructs:

FOR	<line number> FOR <variable>=<expression> TO <expression>

[STEP<expression>]

85  FOR  X=1 TO 1000

90  FOR  X=A TO  B+C  STEP 10

95  FOR  X=100  TO  0  STEP  -1

The FOR  NEXT statements are what is known as a deterministic

looping construct because the number of times the loop will be executed

is determined at the start of the loop when the FOR statement is

executed.  When a FOR statement is executed all instructions between it

and the matching NEXT will repeatedly execute until one of two

conditions  is met.  Each pass through the loop the STEP value is added

to the value of the control variable.  If the STEP value is positive,

the loop will be executed again if the control variable is less than or

equal to the value of the expression following TO.  If the step value is

negative the loop will be executed again if the control variable is

greater than or equal to the value of the expression following the TO.

Note that if no STEP value is supplied (it's optional) that a value of

one (+1) is assumed.  Also note that all of the expressions in the FOR

statement are evaluated only once at the start of the loop.  This means

that that the terminating value and the step value may not be changed in

the body of the loop, however; since the control variable is the same as

any other variable, its value may be changed within the body of the

loop.  This would allow for exiting the loop before it normally would.

Again note that the test of the control variable against the terminating

value is actually performed when the NEXT statement is executed, so the

code between FOR and NEXT will be executed at least once.

FOR - NEXT statements may be nested but they must each use their

own separate control variable.  Currently the maximum number of nested

FOR - NEXT loops is eight (8).  Loops may be exited early by use of

GOTO's however this is not good programming practice and is not

recommended.

NOTE: In the above examples a space follows each of the

expressions in the FOR statement.  This IS REQUIRED so that BASIC11 will

know  where the expression ends.  Failure to follow each expression with

a space will result in an Error being reported (most likely Error #6).

NEXT	<line number> NEXT <variable>

100 NEXT  X

The NEXT statement is used in programs to complete a FOR loop.

The variable specifyed in the NEXT statement must be the same as the

control in the matching FOR. If it is not, Error #36 will be issued and

program execution will stop.  As mentioned above, the test to see

whether the loop should be terminated or not is actually performed when

the NEXT statement is executed.

WHILE	<line number> WHILE  <expression>

500  WHILE  X<=10000

The WHILE - ENDWH statements are considered to be a

non-deterministic type of looping construct because the number of times

the loop will execute is not determined at the start of the loop.  In

fact since the expression following the WHILE statement is evaluated at

the start of the loop, the loop may never be executed if the expression

is false (evaluates to zero) upon entry of the loop.  There is one

important point that needs to be made about the WHILE looping construct.

The statements within the loop must contain a statement that changes the

value of the test expression following WHILE so that the expression

eventually becomes false otherwise the loop will never terminate and the

statements bounded by WHILE and ENDWH will execute forever!

The WHILE statement may be used as part of a multiple statement

line, however; in order to provide improved program readability and to

show the structure of the program this practice is discouraged.

WHILE - ENDWH loops may be nested up to eight (8) levels deep.

WHILE loops may be exited early by use of GOTO's however this is not

good programming practice and is not recommended.

ENDWH	<line number>  ENDWH

600 ENDWH

The ENDWH statement is used only in conjunction with a matching

WHILE statement to enclose a group of lines within a loop.  The effect

of the ENDWH statement is to evaluate the expression following WHILE to

determine whether the loop should be executed again.  Note that the

ENDWH statement may be part of a multi-statement line however, it must

be the first statement on the line.

4.6 Program Termination:

STOP	<line number> STOP

1000  STOP

The STOP statement is essentially a software break (control-C)

instruction. When the STOP statement is executed, program execution is

temporarily suspended and the message:

STOPPED  AT  LINE  #  <line number>

is printed on the terminal.  In the above example <line number>

would be 1000. If no alterations are made to the program after it has

been suspended, execution may be restarted with the CONT command.  The

first statement executed will be the one immediately following the STOP

statement.

END	<line number>  END

300  END

The END statement is used to terminate program execution.  It

does not have to be the last statement and may appear anywhere in the

program.  In fact an end statement need not appear anywhere in the

program.  If BASIC11 tries to execute past the end of the program, an

END statement will automatically be executed.  Unlike the STOP

statement, after an END statement has been executed the program may not

be restarted via the CONT command.

4.7 Real Time Event Statements:

In any control environment, events usually occur asynchronously

with respect to main program execution.  To cope with this kind of

environment the MC68HC11 was designed with an extensive interrupt

structure to support all of its on chip subsystems.  The following

statements all provide control of interrupt driven events directly from

BASIC11.

ONTIME	<line number> ONTIME <expression>,<line number>

25  ONTIME  120,500

35  ONTIME  HR+1,200

95  ONTIME  0,500

In many control situations it is necessary to take periodic

measurements or record certain events at fixed time intervals.  The

ONTIME statement frees the main program from having to continuously

check the value of the system variable TIME in order to determine when

to take a measurement or record an event.  The ONTIME statement allows

program control to be transferred directly to an interrupt handling

routine begining at <line number> when the value of <expression> matches

the value of the system variable TIME. The value of <expression> may

evaluate to any legal integer, however; if <expression> evaluates to

zero (0) it has the effect of disabling the ONTIME function.

One of two methods may be used to generate periodic interrupts

using the ONTIME statement.  The first method involves zeroing the

system variable TIME in the interrupt handling routine with the

statement TIME=0.  This method may be used if continuous timekeeping is

not required by the system.  The second method involves executing the

ONTIME statement in the interrupt routine, adding the desired time

interval (in seconds) to the current value of the system variable TIME.

This second method should be used if continuous timekeeping is required

by the system.  The following examples should clarify things.

First Method:

10  TIME=0

20  ONTIME  10,100

.

.

.

100  TIME=0

.

.

150  RETI

The above example will produce a timer interrupt every 10 seconds.

Second Method:

10  TIME =0

20  ONTIME  20,500

.

.

.

500  ONTIME  TIME+20,500

.

.

550  RETI

The above example will produce a timer interrupt every 20 seconds.

ONIRQ	<line number>  ONIRQ  <expression>,<line number>

10  ONIRQ  1,355

25  ONIRQ  MD,225

The ONIRQ statement allows BASIC11 to directly handle interrupts

that are generated by an active transition on the MC68HC11's IRQ pin.

The <expression> following the ONIRQ keyword is used to select the mode

of the statement.  If the expression evaluates to any non-zero integer,

the servicing of the IRQ interrupt by BASIC11 is enabled.  If the

expression evaluates to zero (0), IRQ interrupts are effectively

disabled.  The <line number> following the expression may be any legal

BASIC11 line number.

ONPACC	<line number>  ONPACC <expression>,<expression>,<line number>

105  ONPACC  1,0,1000

255  ONPACC  A,B,3000

The ONPACC statement allows the programmer to handle events

associated with the MC68HC11's Pulse Accumulator on an interrupt basis.

The first expression following the ONPACC keyword is used to set the

operating mode of the pulse accumulator. The expression must evaluate to

a number from 0 through 4.  The operating modes of the pulse accumulator

are described in the table below.

Mode	Action On Clock

0	Disables the Pulse Accumulator

1	Falling Edge on PA7 Increments the Counter

2	Rising Edge on PA7 Increments the counter

3	A "0" on PA7 Inhibits E�64 from Incrementing Counter

4	A "1" on PA7 Inhibits E�64 from Incrementing Counter

The second expression is used to determine which of two events

will cause an interrupt to be generated by the pulse accumulator.  If

the expression evaluates to zero (0) then an interrupt will be generated

each time an active edge is detected on PA7 as described in the table

above.  If it evaluates to 1, the pulse accumulator will generate an

interrupt only when it overflows from $FF to $00.  The <line number>

tells BASIC11 where the interrupt routine begins when a Pulse

Accumulator interrupt occurs.

For more information on the MC68HC11's Pulse Accumulator

subsystem, please refer to the parts data sheet.

RETI	<line number>  RETI

485  RETI

All BASIC11 interrupt service routines must end with this

statement. Failure to  end an interrupt routine with RETI will result in

all successive interrupts being masked!  This will effectively stop the

system TIME function.

4.8 Miscellaneous Statements:

DIM	<line number>  DIM  <subscripted variable> [,subscripted variable....]

10  DIM  AX(100), DX(9), LK(1000)

20  DIM  Z(A+5), D(X)

30  DIM  X(0)

The DIM statement, which was discussed briefly in section 2.3

(Variables), is used to allocate storage for subscripted variables when

a program is run.  As can be seen from the example in line 20 above, the

expression in parenthesis does not have to be a constant. This is

because array storage is dynamically allocated at runtime.  This feature

is especially nice in control applications where memory is usually at a

premium because large arrays don't have to be dimensioned in advance to

fit the worse case.  All subscripted variables must appear in a DIM

statement before they may be used in an expression. Failure to do this

will result in Error # 24 being issued when the variable is referenced.

The storage required by subscripted integer variables is:

2*(<expression>+1)+2     bytes

Remember that all subscripts start at zero. In the example in

line 10 above, the variable AX(100) would actually create 101 integer

variables, AX(0) through AX(100). Although it may seem strange the

example in line 30 is legal.  This will create a single integer

subscripted variable X(0).

POKE	<line number>  POKE(<expression>,<expression>)

45  POKE($ED00,$5A)

55  POKE(AD,X*5)

The POKE statement allows the BASIC11 program to directly modify

RAM memory or I/O locations.  The first expression within the

parenthesis is the address at which the second expression will be

stored.  The first expression may evaluate to any legal integer number.

However the second expression must be in the range  0 � expression � 255

since a byte location is being written to.  If the second expression is

outside the above range, Error #48 will be issued.  Care should be taken

when using this statement so that part of the BASIC11 program or its

data are not overwritten.

REM	<line number>  REM  [any text]

10  REM  THIS IS A REMARK STATEMENT

The REM statement is used to insert comments about the operation

or structure of  a program.  Any text following the REM statement is

ignored, so if it appears in a multiple statement line, it should be the

last statement on the line.  If control is passed to a REM statement by

a GOTO, GOSUB, etc. , control is just passed to the line following the

REM statement.

TRON	<line number>  TRON

20  TRON

The TRON statement is used to turn the trace mode on.  The trace

mode, when turned on, will print line numbers in brackets as each line

is executed.  This can be used as an aid in debugging programs.

TROFF	<line number>  TROFF

100  TROFF

The TROFF statement is used to turn the trace mode off.

5.0 Built in Functions:

BASIC11 has a number of built in functions that are used to

perform common operations on numerical quantities, perform special

calculations, call user written assembly language subroutines, and

access some of the special hardware features of the MC68HC11.

5.1 Mathematical Functions:

ABS(X)	The ABS function will return the ABSolute value of the

expression in parenthesis.  The function will always return a positive

number as its result.

FDIV(X,Y)	The FDIV function is used to perform an unsigned

fractional divide using the MC68HC11's FDIV instruction.  This function

allows BASIC11 to resolve fractional parts of the remainder of an

integer divide without using floating point math.  The result is a

binary weighted decimal number.  Some examples may clarify what the

function does.

3 / 4 = .75  decimal

3 / 4 = $C000  binary weighted decimal

2 / 4 = .50  decimal

2 / 4 = $8000  binary weighted decimal

1 / 4 = .25  decimal

1 / 4 = $4000  binary weighted decimal

.99999 = $FFFF

For the function to execute properly X must be less than Y and Y

may not be equal to zero.

If either condition exists Error #44 will be issued and program

execution will terminate.

RND(X)	The RND function will return a pseudo random number between 0

and 32767 inclusive. The value of the argument X has the following

effect on the function:

For x<0 a new series of random numbers will be started by

reading the current value of the timer/counter and using it as the new

seed value.

For X=0 a new random number will be returned each time the

function is called.

For X>0 the last random number that was generated is returned.

Note that the function only generates pseudo random numbers and

that a particular series will repeat every 65536 calls of the function.

SGN(X)	The SGN function will return a plus one (1) if the argument is

positive, zero (0) if the argument is zero, and a minus one (-1) if the

argument is negative.

5.2 Print Functions:

CHR$(X)	The CHR$ function will return a single character string whose

ASCII value is the argument X.  This function is very useful for sending

non-printable ASCII characters to an output device.  The value of the

argument X must be in the range of 0 � X � 255 or Error #43 will be

issued.  This function may only be used in the PRINT statement.

HEX(X)	The HEX function is used to convert a binary number to a four

digit hexidecimal string. This function is very useful when printing the

contents of memory locations or I/O ports. This function may only be

used in the PRINT statement.

HEX2(X)	The HEX2 function performs a similar operation to the HEX

function except that it is used to convert a number in the range 0 � X �

255 to a two digit hexidecimal string. If a number outside the specifyed

range is passed as an argument to the HEX2 function, Error #50 will be

reported.

TAB(X)	The TAB function will move the cursor to column X on the output

device. If the output device is already past column X then no action is

performed.  The argument to the TAB function must be in the range of 0

<= X <= 255 or Error # 42 will be issued. This function may only be used

in the PRINT statement.

5.3 Hardware Related Functions:

ADC(X)	The ADC function allows a program to directly access the

MC68HC11's on board 8-bit A-to-D converter.  Any one of the eight

channels may be read by calling the function with the proper argument.

If the argument is not in the proper range (between 0 and 7) Error #45

will be issued.  The A-to-D converter is operated in the single channel

mode and is converted four times.  These four conversions are then

averaged by BASIC11 and the result is then returned.  Since the A-to-D

conversion time is fast (16�s at 2.0 MHz) this tends to help average out

any noise in the reading.

CALL(X)	Even though BASIC11 is extremely fast for an interpreted BASIC,

there are still some things that may need to be controlled that it can't

keep up with.  The CALL function allows machine language subroutines to

be called directly from BASIC11.  The CALL function must appear in an

expression since it will return a 16-bit number as a result of the

function call.  Some examples follow:

10  F=CALL($EAF0)

20  Z=CALL(AX*2)

30 PRINT  CALL($100)

The users machine language program must only preserve the

Y-index register, the stack pointer, and the  current state of the

stack.  All other registers may be destroyed.  The users subroutine is

entered via a  JSR  (Jump to SubRoutine) instruction, therefore it must

end with the execution of an  RTS  (ReTurn from Subroutine) instruction.

Generally the users subroutine should have about 100 bytes of stack

space available.  If more than this is needed, the subroutine will have

to allocate its own stack storage space.

EEP(X)	As mentioned in section 4.1 the EEP statement allows a BASIC11

program to directly write information to the MC68HC11's on chip EEPROM

when the EEP statement appears to the left of the equals as a basic

"statement".  When EEP appears on the right side of the equals it

syntaticly will act like a function and will return the value currently

stored in the EEP array location described by the argument X.  Again,

the argument to the function is actually the subscript of the EEP array

so it may not be negative and must not be greater than the maximum value

of 255.

PEEK(X)	The PEEK function performs the opposite action of the POKE

function. It allows BASIC11 to directly retrieve the contents of any

memory or I/O location in the MC68HC11's memory map.  The argument X,

since it is an address, is taken to be an unsigned number so X may take

on any integer value.  A single byte is returned by the function so its

value will be �0 and � 255.

PORTA

PORTB

PORTC

PORTD

PORTE	The PORTx functions are different from the other functions in

that they do not require an argument.  Essentially these functions act

as special variables that allow direct reading of the MC68HC11's I/O

ports from BASIC.

PORTC and PORTD are general purpose I/O ports and as such may

have each pin of the port programed as either an input or an output.

Each ports Data Direction Register (DDR) is used to specify the primary

direction of data on the I/O pin.  If the corresponding port pins DDR

bit is set to a one (1) the port pin will be configured as an output.

If the DDR bit is cleared to a zero (0) the port pin will be configured

as an input and will become high impedance.  When a bit which is

configured for output is read, the value returned is the value at the

input to the pin driver.  If a write is executed to a pin that is

configured as an input, the value will be stored in an internal latch so

that if the pin is later configured as an output, the latched value will

then appear on the output

PORTA, PORTB, and PORTE are all fixed direction Ports with the

exception of bit-7 of Port A.  When PORTB is being used for general

purpose outputs, it is configured for output only and reads return the

actual level sensed at the input of the pin drivers.  When PORTA is

being used for general purpose I/O, bits 0,1, and 2 are configured as

inputs and writes to these bits have no effect or meaning.  Bits 3, 4,

5, and 6 are configured for output only and reads return the actual

level sensed at the input of the pin drivers.  Bit 7 of PORTA can be

configured as either an input or an output via the DDRA7 bit in the

PORTA control register (PACTL).  PORTE contains the eight inputs to the

A-to-D converter, however they may also be used as digital inputs.

Writes to the PORTE address have no meaning or effect.

For a more complete discussion of the function of the I/O

subsystems contained in the MC68HC11, it is suggested that the parts

data sheet be consulted.

TIME	Like the PORTx functions, the TIME function requires no

arguments and is used to retrieve the current value of the system time.

PACC	When the keyword PACC appears to the right of the equals sign it

allows the program to retrieve the current value of the Pulse

Accumulator.  Effectively  PACC is a function that requires no

arguments.

6.0 Error Reporting:

BASIC11 has an extensive error reporting structure that reports

two basic types of errors.  The first category is command line errors.

If a mistake is made by either typing an illegal command or a syntax

error is detected either in a program line or a statement that is to be

executed in the direct mode, BASIC11 will print the contents of the

input buffer.  On the next line asterisks and arrows will be printed

showing the approximate location of the error within the line.  Finally,

a number is printed telling the operator what is wrong with the line. In

the  example shown below programmer input is underlined.

#10  FOR  X=1  100  STEP  2

10  FOR  X=1  100  STEP  2

*********^^^

ERROR  #17

READY

#

Looking up error number 17 in the error table we find that we

have inadvertently left out the "TO" in the FOR statement.  By retyping

the line with "TO" between the 1 and 100 BASIC11 will accept the line.

When the programmer mistypes a command, Error number 3 (Invalid

Expression) will generally be issued.  An example follows.

#LOST          (what the programmer meant to type was LIST)

LOST

*^^^

ERROR  #3

READY

#

The reason  error number 3 is issued is that BASIC11 first

searches its command table to see if the programmer has typed a command.

If no match is found, BASIC11 then searches its statement table to try

to match the input buffer with one of the keywords.  If no match is

found, BASIC11 assumes that the statement is an implied LET.  In the

above example the first two letters, "LO", would be assumed to be a

variable name, and the rules say that in an implied (or declared) LET

the assignment variable must be immediately followed by an equals ("=").

The second category of errors is runtime errors.  These errors,

which are context dependent, occur while the program is running.  All

runtime errors are considered to be fatal in BASIC11 and will

immediately terminate program execution.  A message will be printed on

the terminal indicating what error occurred and in which line it

occurred.  Even though BASIC11 does not list the source line for runtime

errors, the error number is specific enough that the problem can easily

be identified.

A list of error numbers and their meanings follows.

Error #	Meaning

1	Line number < 0 or > 32767

2	Syntax Error

3	Invalid Expression

4	Unbalanced Parenthesis

5	Data Type Mismatch

6	Illegal Operator

7	Illegal Variable

8	Illegal Token

9	Out of Memory

10	Integer Overflow

11	Invalid Hex Digit

12	Hex Number Overflow

13	Missing Quote

14	Missing Open or Closing Parenthesis

15	Syntax Error in "ON" Statement

16	Missing "THEN" in an "IF" Statement

17	Missing "TO" in a "FOR" Statement

18	Line Number Zero (0) Not Allowed

19	Illegal Data Type

20	Expression Too Complex

21	Missing Comma

22	Missing Comma or Semicolon

23	Math Stack Overflow

24	Undimensioned Array

25	Subscript Out of Range

26	Divide By Zero

27	Line Number Not Found

28	Too Many Nested "GOSUB's" (maximum is eight)

29	"RETURN" without "GOSUB"

30	Too Many Active "WHILE's" (maximum is eight)

31	"ENDWH" without "WHILE"

32	"ON" argument is Negative, Zero, or Too Large

33	Non-subscriptable Variable Found in "DIM" statement

34	Variable has Already Been DIMensioned

35	Too Many Active "FOR - NEXT" loops (maximum is eight)

36	Mismatched "FOR - NEXT" loop

37	Can't Continue

38	Out of Data in "READ" or "RESTORE" Statement

39	Negative Subscripts Not Allowed

40	"EEP()" Subscript Negative or > 255

41	Function Only Allowed in "PRINT" Statement

42	Argument < 0 or > 255 in "TAB()" Function

43	Argument < 0 or > 255 in "CHR$()" Function

44	Overflow or Divide by Zero in "FDIV()" Function

45	Invalid Channel Number in "ADC()" Function

46	Tried to Assign a Value of < 0 or > 255 to a PORT

47	Illegal PORT

48	Illegal Device Number

49	Uninitalized I/O Vector

50	Argument < 0 or > 255 in "HEX2()" Function

51	Statement not allowed in immediate mode

52	RETI executed when not in an interrupt routine

53	Tried to assign a value of <0 or >255 to PACC

54	Interrupt or Count mode error in ONPACC

Appendix A

This appendix contains information specific to BASIC11's

implementation for the Motorola MC68HC11 evaluation board (Part #

M68HC11EVB).

Program Storage EEPROM:

The M68HC11EVB was designed to be an extremely low cost way of

evaluating the capabilities of the MC68HC11 microcomputer hence it has

no memory expansion capabilities.  To be able to save BASIC11 programs

either while working on them or for stand alone execution in a control

environment, some additional non-volatile memory must be added.  This

memory is added in the form of an 8K x 8 EEPROM that is piggybacked on

to the 8K x 8 RAM (U4).  The EEPROM that is used must have latched

addresses, data, control signals, automatic erase before write

capabilities, and have self timed write cycles. Several parts that meet

these requirements are the XICOR X2864A-25, Samsung KM2864A, and EXEL

XL2864A-25.

To add the 8K x 8 EEPROM it is necessary to parallel all

address, data, and control lines of the part except pin #20 (chip

enable), pin #22 (output enable), and pin #1 (RDY/BUSY). Do this by

placing the EEPROM on top of the 8K x 8 RAM (U4) and carefully soldering

all the leads, except pin 20, pin 22, and pin 1, together.  Carefully

bend pin #20, #22 and #1 of the EEPROM away from the corresponding pins

of the RAM I.C., U4.  Pin #1 may be cut off since it will not be used.

You may piggyback a socket instead of directly soldering the EEPROM to

the RAM.  Sockets are cheaper than EEPROMS!

To meet the write cycle timing requirements of the 2864 EEPROMS,

the output enable must be negated (logic 1) during writes.  To

accomplish this, an inverted R/W signal is routed to pin #22 of the

EEPROM.  On the bottom of the board, cut the trace connecting pin #11

and pin #13 of U11 (MC74HC14).  Using a piece of small gauge wire,

connect pin #11 of U11 (MC74HC14) to pin #10 of U6 (MC14066B).  Once

again using a piece of small gauge wire, connect pin #10 of U11 to pin

#22 of the EEPROM.

Next, using a piece of wire wrap or other small gauge wire,

connect pin #20 of the EEPROM to pin #59 of the MCU I/O Port Connector,

P1.  Again, using a piece of wire wrap or other small gauge wire,

connect pin #60 of connector P1 to pin #12 of the 74HC138 (U5).  This

connects the chip enable pin of the EEPROM to the address decoder and

places it in the memory map from addresses $6000 thru $7FFF. As a final

step, solder a 10k � pull-up resistor between pins #12 and #16 of the

74HC138 (U5).

Finish the installation by placing a fabricated jumper between

pin #59 and 60 of the port connector P1.

The fabricated jumper installed in the previous step is used so

that the EEPROM may be disabled during powerup.  This will effectively

disable the automatic execution of a program stored in it if the AUTOST

command was entered prior to the last power down.

System Printer:

BASIC11 contains device drivers that allows the connection of a

serial printer to the HOST port of the M68HC11EVB.  Since the HOST port

on the HC11EVB does not support any hardware handshaking, the printer

must be able to support  X-On/X-Off  software handshaking.  The

characters used for X-On/X-Off flow control are currently set to $13 for

X-OFF and $11 for X-ON. If these characters need to be changed to

accommodate other printers or devices, there are two variables located

on page zero that may be changed by POKEing the desired values into

them.  The X-ON variable is located at $0043 and the X-OFF variable is

located at $0044.  The default configuration for the printer port is

9600 BAUD, eight data bits, one start bit, and one stop bit.  This may

easily be changed by poking the proper value into the BAUD control

register located at $102B.  For further information on the MC68HC11's

Serial Communications Interface (SCI), please refer to the parts data

sheet.

Besides being used to get hard copy listings of programs with

the LLIST command, the system printer may be accessed from within a

program by using the "PRINT#<expression>" form of the PRINT statement

with the value of <expression> evaluating to 1.  So the program line 10

PRINT #1, "HELLO THERE WORLD", would print "HELLO THERE WORLD" on the

system printer.  Even though it is not possible to "INPUT" anything from

the system printer, the input vector for device #1 has been setup to

allow characters to be received via the RxD line of the target system.

One final note about using the SCI port of the HC11 for serial

Input and Output.  The HC11EVB hardware was designed so that both the

TxD and RxD lines of the SCI could be disconnected from the HOST

computer I/O connector.  The TxD line is disconnected from the HOST port

by simply removing the fabricated jumper, J4.  The RxD line is

alternately connected to either the target system or HOST port by means

of an MC74HC4066 digital switch and an MC74HC74 flip-flop.  The

switching of the receiver line from the target system to the HOST I/O

port is accomplished by writing a logic one in bit 0 to any address in

the range $A000-$BFFF.  Likewise, writing a logic zero in bit 0 to any

address in the same range results in the target system being connected

to the RxD pin of the HC11.

BASIC11 initializes the hardware so that the SCI RxD line is

connected to the HOST I/O port. Connection to the target system may be

accomplished by executing the statement POKE($A000,1).

Alternate Device I/O:

BASIC11 supports a form of device independent I/O in the PRINT,

INPUT, and INBYTE statements by using the "#<expression>" modifier after

each keyword.  The expression is evaluated and is used as an index into

a pair of vector tables located in page 0 RAM.  Each table consists of

eight entries of two byte addresses, therefore <expression> must

evaluate to be in the range of 0 through 7.  The two tables, "INTABLE"

AND "OUTABLE",  are located at $00A4 and $00B4 respectively. Currently

device #0 is set up to be the control console I/O routines and device #1

is setup to send data to a system printer connected to the HOST I/O

port.  Device numbers 2 through 7 are available for use by the

programmer.

As mentioned above, device #0 is used for the control console

and is currently set up to use the MC6850 ACIA for system I/O.  If it is

desired to use other hardware for system I/O the user may replace the

current vectors in INTABLE and OUTABLE.  If control-C break in

capability is to be maintained for the new hardware, two additional

vectors must be modified.

The CONSTAT (CONsole STATus) vector located at $009E should

point to a routine that checks the control console hardware to see if a

character has been typed.  If a character has been received,the routine

should return a NOT EQUAL condition (Z condition code bit cleared). This

routine MAY NOT modify any registers except the condition code register.

The CONINNE (CONsole INput No Echo) vector located at $00A1 should point

to a routine that gets a character from the console hardware but does

not echo it back to the hardware.  Note that these vectors are three (3)

byte vectors with the first byte containing a JMP instruction ($7E). The

address of the user routines needs to be put in the second and third

bytes.

By using the CONSTAT vector to point to a routine that always

returns an equal condition, (Z-bit in the condition code register set)

the control-C break-in capability can effectively be disabled.

Note that any additional device drivers must be provided by the

programmer and any hardware initializations must be performed before the

"device" is used.

Interrupt Vector Table:

All twenty of the interrupt vectors for the different subsystems

on the MC68HC11 are located in the memory map at locations $FFD4 through

$FFFF.  To provide for more flexibility in using the subsystems in an

interrupt driven mode, the ROM hardware vectors "point" to a second

"JUMP" vector table located in RAM on page zero.  The table, as shown

below, may be altered by the programmer to point to special interrupt

handlers for a particular application.  The PACCIE, PACCOVF, TOC1, and

IRQI vectors are initialized by BASIC11 to point to its own interrupt

routines for the various real time control functions provided by

BASIC11.  The ILLOP, COP, and CMF vectors are initialized to jump to the

start of BASIC11. All the rest of the vectors point to an RTI

instruction.

*

*

ORG	$009E

*

CONSTAT	RMB	3	GET CONTROL CONSOLE STATUS (FOR BREAK ROUTINE).

INCONNE	RMB	3	GET BYTE DIRECTLY FROM CONTROL CONSOLE,NO ECHO.

*

*

ORG	$00A4

*

INTABLE	RMB	16	RESERVE SPACE FOR 8 DIFFERENT INPUT ROUTINES.

OUTABLE	RMB	16	RESERVE SPACE FOR 8 DIFFERENT OUTPUT ROUTINES.

*

*

*

*

ORG	$00C4    START OF RAM INTERRUPT VECTORS.

*

RAMVECTs	 EQU	*

SCISS	RMB	3	SCI SERIAL SYSTEM.

SPITC	RMB	3 	SPI TRANSFER COMPLETE.

PACCIE	RMB	3	PULSE ACCUMULATOR INPUT EDGE.

PACCOVF	RMB	3	PULSE ACCUMULATOR OVERFLOW.

TIMEROVF	RMB	3	TIMER OVERFLOW.

TOC5	RMB	3	TIMER OUTPUT COMPARE 5.

TOC4	RMB	3	TIMER OUTPUT COMPARE 4.

TOC3	RMB	3	TIMER OUTPUT COMPARE 3.

TOC2	RMB	3	TIMER OUTPUT COMPARE 2.

TOC1	RMB	3 	TIMER OUTPUT COMPARE 1.

TIC3	RMB	3	TIMER INPUT CAPTURE 3.

TIC2	RMB	3	TIMER INPUT CAPTURE 2.

TIC1	RMB	3	TIMER INPUT CAPTURE 1.

REALTIMI	RMB	3	REAL TIME INTERRUPT.

IRQI	RMB	3	IRQ INTERRUPT.

XIRQ	RMB	3	XIRQ INTERRUPT.

SWII	RMB	3	SOFTWARE INTERRUPT.

ILLOP	RMB	3	ILLEGAL OPCODE TRAP.

COP	RMB	3	WATCH DOG TIMER FAIL.

CMF	RMB	3	CLOCK MONITOR FAIL.

*

*

##########################